2000W Load Controller

Hardware Design Assignment — Gray Mobility

Designed by: Sujeev Ravindra Swami  |  Tool: KiCad 8 + LTspice  |  Date: April 2026

# 1. Circuit Design Overview

This project presents a complete hardware design for a 2000W DC load controller intended for electric vehicle applications. The system accepts a 40V DC input supply and drives a high-power LED load operating at 36V / 56A (peak 100A) using a switched MOSFET topology controlled by a 32-bit STM32 microcontroller.

## Power Stage

The LM2596S-3.3 buck converter steps the 40V input down to a regulated 36V rail for the load and feeds the LM1117DT-3.3 LDO regulator which provides 3.3V for the microcontroller and gate driver. The IRF540N N-channel MOSFET (100V, 33A continuous, 110A pulsed) is used in parallel configuration to handle the 56A continuous and 100A peak load current requirement. Feedback resistors R_fb1 (1.21KΩ) and R_fb2 (3.3KΩ) set the LM2596 output voltage to 36V.

## Gate Drive Circuit

The ULN2003A Darlington array acts as a gate driver, accepting the 3.3V PWM signal from STM32 pin PA8 and driving the MOSFET gate with sufficient current. A 10Ω gate resistor (R_gate) limits inrush current and reduces switching noise. A 36V Zener diode (D4) clamps the gate-source voltage to prevent MOSFET gate oxide damage.

## Temperature Sensing

Four NTC thermistors (Murata NCP18XH103F03RB, 10K at 25°C) are connected in voltage divider configuration with 10KΩ pull-up resistors to the 3.3V rail. The midpoint voltage is read by STM32 ADC pins PA0-PA3. The firmware compares these readings against temperature thresholds and disables the MOSFET if overtemperature is detected, protecting both the load and PCB.

## Protection Mechanisms

Overvoltage protection is implemented using Zener diodes on all three power rails: D1 (39V) on the 40V input, D2 (36V) on the output rail, and D3 (3.3V) on the MCU rail. Overcurrent protection uses a 0.01Ω shunt resistor (R_shunt) with the voltage drop read by STM32 PA4 — allowing firmware to calculate instantaneous current and shut off the MOSFET if the 100A peak limit is exceeded. A 100A blade fuse (F1) provides hard protection as a last resort.

## Microcontroller

The STM32F103C4Tx 32-bit ARM Cortex-M3 microcontroller manages all system functions: generating 10kHz PWM on PA8 to control MOSFET switching, reading 4 NTC temperature sensors via ADC, monitoring load voltage on PA5, monitoring load current on PA4, and implementing protection logic. The NRST pin is held high via a 10KΩ pull-up resistor and BOOT0 is tied to GND for normal flash boot operation.

# 2. Design Assumptions

Load Assumptions:

Load voltage = 36V, continuous current = 56A, peak current = 100A (10ms max). Total power = 36V x 56A = 2016W ≈ 2000W.

MOSFET Selection:

IRF540N rated at 100V Vds and 33A Id. For 56A continuous, multiple MOSFETs in parallel are assumed in production. Simulation uses single MOSFET for verification.

Firmware Assumptions:

PWM frequency = 10kHz. ADC sampling rate = 1kHz for temperature, 10kHz for current. Overcurrent threshold = 100A. Overtemperature threshold = 85°C.

PCB Assumptions:

2-layer FR4 PCB, 1.6mm thickness, 2oz copper weight for high current handling. Power tracks 3mm wide. Signal tracks 0.5mm. GND copper pour on bottom layer.

# 3. Bill of Materials (BOM)

All components selected for cost-effectiveness, availability in India, and reliability for EV applications. Total estimated component cost: ₹400-500 per board.

| Ref | Part | Description | Qty | Package | Manufacturer | Est. Cost |
| --- | --- | --- | --- | --- | --- | --- |
| U1 | LM2596S-3.3 | Buck Converter IC | 1 | TO-263-5 | Texas Instruments | ~₹45 |
| U2 | STM32F103C4Tx | 32-bit Microcontroller | 1 | LQFP-48 | STMicroelectronics | ~₹120 |
| U3 | ULN2003A | Gate Driver / Darlington Array | 1 | DIP-16 | Texas Instruments | ~₹15 |
| U4 | LM1117DT-3.3 | 3.3V LDO Regulator | 1 | SOT-223 | Texas Instruments | ~₹25 |
| Q1 | IRF540N | N-Channel Power MOSFET | 1 | TO-220 | Vishay/IR | ~₹35 |
| J1 | Conn_01x02 | 40V DC Input Connector | 1 | PinHeader 2.54mm | Generic | ~₹5 |
| J2 | Conn_01x02 | Load Output Connector (60A rated) | 1 | XT60 / Anderson | Generic | ~₹25 |
| F1 | Fuse 100A | Blade Fuse - Overcurrent Protection | 1 | Fuse_1812 | Littelfuse | ~₹20 |
| D1 | 1N5349B | 39V Zener - OVP Input Rail | 1 | DO-35 | Vishay | ~₹8 |
| D2 | 1N5348B | 36V Zener - OVP Output Rail | 1 | DO-35 | Vishay | ~₹8 |
| D3 | BZX55C3V3 | 3.3V Zener - MCU Rail Protection | 1 | DO-35 | Vishay | ~₹5 |
| D4 | 1N5348B | 36V Zener - Gate Drive Protection | 1 | DO-35 | Vishay | ~₹8 |
| R_gate | 10Ω 1/4W | MOSFET Gate Resistor | 1 | R_0805 | Generic | ~₹2 |
| R_shunt | 0.01Ω 2W | Current Sense Shunt Resistor | 1 | R_2512 | Bourns | ~₹15 |
| R_feedback | 10KΩ 1/4W | Voltage Feedback Resistor | 1 | R_0805 | Generic | ~₹2 |
| R_fb1 | 1.21KΩ 1/4W | LM2596 Feedback Resistor 1 | 1 | R_0805 | Generic | ~₹2 |
| R_fb2 | 3.3KΩ 1/4W | LM2596 Feedback Resistor 2 | 1 | R_0805 | Generic | ~₹2 |
| R_nrst | 10KΩ 1/4W | STM32 Reset Pull-up Resistor | 1 | R_0805 | Generic | ~₹2 |
| NTC1-4 | NCP18XH103F03RB | 10K NTC Temperature Sensor | 4 | R_0805 | Murata | ~₹15 each |
| C1,C2 | 10µF 50V | Decoupling Capacitors | 2 | C_0805 | Generic | ~₹5 each |

Note: Prices are approximate retail estimates (Bangalore/online). Bulk pricing will reduce costs significantly. XT60 or Anderson connector recommended for J2 due to high current rating requirement.

# 4. Simulation Results

LTspice simulation was performed to verify MOSFET gate drive circuit operation. The IRF540N was driven with a 10kHz PWM signal (0-5V) through a 10Ω gate resistor from a simulated ULN2003 output. Simulation confirmed correct switching behavior:

Gate voltage: Clean 0V to 5V switching at 10kHz with 50% duty cycle

Drain voltage: Switches between 36V (OFF) and 0V (ON) as expected

Load current: Pulses to 56.25A (36V / 0.64Ω load resistor) confirming correct power delivery

Designed for Gray Mobility Hardware Engineering Internship Assignment  |  Tools: KiCad 8, LTspice XVII  |  Standard: ISO 26262 aware design